Chat protocol.

The protocol is designed to be useable both by humans and by user

interface programs.

Clients connect to the chat server using a tcp connection.  The

connection is handled in a full-duplex fashion - Both the server and

the client may submit messages at any time.

The chat server prefixes all administrative messages with a

three-digit positive number, a space and the message.  An exception to

this rule are user messages (i.e. text entered from one participant to

one or multiple other receipients).  These are prefixed by the nick

name of the sender, enclosed in brackets ('<' and '>') for public

messages or asterisks ('*') for private messages.

Coded server messages may have parameters.  The parameters follow the

space, and may themselves be followed by comments.  The parameters and

the comments are delimited by a space character.  This makes additions

of parameters to messages impossible.

The following messages are defined for the server:

100 <message...>

Informational message for human consumption

110 <channel> <nick> <fromhost>

User status summary

111 <date> <time> <code> <message>

Server log information

112 <information>

Server user information

113 <ignore-info>

Ignore command confirmation

114 <nick> changed the channel topic to <topic>

Channel topic change confirmation

115 <channel> <topic>

Channel status information

116 <nick> <from> <server> <userinfo>

User status information

117 <nick> <statistics>

User statistics information

118 <nick> <action>

User action information

201 <protocol> <server-name>

Server signon

211 <nickname>

User signon

221 <nickname>

User signoff

231 <nickname>

Channel join

232 <nickname>

Channel departure

241 <oldnick> <newnick>

Nickname change

301

Message truncated

302

Message too long

401

Character set failure, syntax error

402

Uninterpretible command

403

Not logged in

404

Already logged in

411

Nickname in use

412

Nickname not found (private message not sent)

413

Channel not found

414

Access violation

501

Disconnected by own request

502

Disconnected by operator

Message continuation is supported by replacing the space after the

message code with a dash ('-') in all but the last lines of a message

When a client connection

has been established, the server responds with a signon message.